4/26/2026

Lab

COMPUTER COMMUNICATIONNETWORKS

Name: Neha Khatri Roll No: TCT-022

Batch: 2023 Department: Computer Science

Lab # 05

# Objective

The objective of this lab is to configure Inter-VLAN routing using the Router-on-a-Stick technique. This enables communication between devices located in different VLANs (VLAN 10 and VLAN 20) through a single physical router interface by utilizing sub-interfaces and VLAN tagging.

# Physical Connections

| Source Device | Port | Destination Device | Port |
| --- | --- | --- | --- |
| PC1 | Fa0 | Switch (LSW2) | Fa0/1 |
| PC2 | Fa0 | Switch (LSW2) | Fa0/2 |
| PC3 | Fa0 | Switch (LSW2) | Fa0/3 |
| PC4 | Fa0 | Switch (LSW2) | Fa0/4 |
| Switch | Fa0/6 | Router (R1) | Fa0/0 |

- Switch Configuration (LSW2)

## VLAN Creation

enable

configure terminal vlan 10

name VLAN10 exit

vlan 20

name VLAN20 exit

  - Port Assignment interface fastEthernet 0/1 switchport mode access switchport access vlan 10 exit

interface fastEthernet 0/2 switchport mode access switchport access vlan 10 exit

interface fastEthernet 0/3 switchport mode access switchport access vlan 20 exit

interface fastEthernet 0/4 switchport mode access switchport access vlan 20 exit

  - Trunk Configuration interface fastEthernet 0/6 switchport mode trunk

switchport trunk allowed vlan 10,20 no shutdown

exit

## Verification Commands

show vlan brief

show interfaces trunk

# Router Configuration (R1)

## Enable Interface

enable

configure terminal interface fastEthernet 0/0 no shutdown

exit

## Sub-interface Configuration (VLAN 10)

interface fastEthernet 0/0.10 encapsulation dot1Q 10

ip address 192.168.10.150 255.255.255.0

exit

## Sub-interface Configuration (VLAN 20)

interface fastEthernet 0/0.20 encapsulation dot1Q 20

ip address 192.168.20.150 255.255.255.0

exit

## Save Configuration

end

write memory

## Verification Commands

show ip interface brief

show running-config | section interface FastEthernet0/0

show ip route

# PC Configuration

Each PC was configured with an IP address, subnet mask, and default gateway according to its VLAN.

# Testing and Verification

## Same VLAN Communication

PC1 successfully communicated with PC2 (0% packet loss), confirming correct VLAN configuration.

# Conclusion

In this lab, Inter-VLAN routing was successfully implemented using the Router-on-a-Stick method.

VLAN 10 and VLAN 20 were created on the switch, and ports were assigned correctly. A trunk link was configured between the switch and router to allow multiple VLANs to pass through a single interface. On the router, sub-interfaces were created and configured using 802.1Q encapsulation.

All devices were assigned proper IP addresses and default gateways. Communication within the same VLAN (intra-VLAN) and between different VLANs (inter-VLAN) was tested and worked successfully.

The traceroute result confirmed that traffic between VLANs passes through the router, which verifies that Inter-VLAN routing is working correctly.