LAB 5

Inter-VLAN Routing

Router-on-a-Stick Configuration

Objective

The objective of this lab is to configure Inter-VLAN routing using the Router-on-a-Stick technique. This approach enables seamless communication between devices residing in different VLANs — specifically VLAN 10 and VLAN 20 — through a single physical router interface. The method leverages sub-interfaces and 802.1Q VLAN tagging to logically separate traffic while sharing one physical uplink between the switch and router.

# Physical Connections

The following table outlines the physical cabling used in this lab topology. All end devices connect to the switch via their FastEthernet 0 interface, and the switch uplinks to the router through FastEthernet 0/6.

| Source Device | Source Port | Destination Device | Destination Port |
| --- | --- | --- | --- |
| PC1 | Fa0 | Switch (LSW2) | Fa0/1 |
| PC2 | Fa0 | Switch (LSW2) | Fa0/2 |
| PC3 | Fa0 | Switch (LSW2) | Fa0/3 |
| PC4 | Fa0 | Switch (LSW2) | Fa0/4 |
| Switch (LSW2) | Fa0/6 | Router (R1) | Fa0/0 |

# Switch Configuration (LSW2)

The switch LSW2 is configured to support two VLANs and to forward tagged traffic toward the router via a trunk link. The configuration is broken into four steps: VLAN creation, port assignment, trunk setup, and verification.

## VLAN Creation

Two VLANs are defined — VLAN 10 and VLAN 20 — each assigned a descriptive name to reflect the network segments they represent.

enable

configure terminal vlan 10

name VLAN10

exit vlan 20

name VLAN20 exit

## Port Assignment

Each access port is associated with the appropriate VLAN. PC1 and PC2 are assigned to VLAN 10, while PC3 and PC4 belong to VLAN 20. Access mode is explicitly set to prevent any unintended trunk negotiation.

| interface fastEthernet | 0/1 |
| --- | --- |
| switchport mode access |  |
| switchport access vlan | 10 |
| exit |  |
| interface fastEthernet | 0/2 |
| switchport mode access |  |
| switchport access vlan | 10 |
| exit |  |
| interface fastEthernet | 0/3 |
| switchport mode access |  |
| switchport access vlan | 20 |
| exit |  |
| interface fastEthernet | 0/4 |
| switchport mode access |  |
| switchport access vlan | 20 |
| exit |  |

## Trunk Configuration

Port Fa0/6 is configured as a trunk to carry both VLAN 10 and VLAN 20 traffic toward the router. Only the required VLANs are permitted on this trunk, which is a recommended security practice.

interface fastEthernet 0/6 switchport mode trunk

switchport trunk allowed vlan 10,20 no shutdown

exit

## Verification Commands

After completing the switch configuration, the following commands are used to confirm that VLANs exist and are correctly assigned, and that the trunk link is operational.

| Command | Purpose |
| --- | --- |
| show vlan brief | Displays all VLANs and their associated access ports |
| show interfaces trunk | Confirms trunk status and permitted VLAN list on Fa0/6 |

# Router Configuration (R1)

Router R1 is configured using the Router-on-a-Stick method. A single physical interface (FastEthernet 0/0) is brought up, and two logical sub-interfaces are created — one per VLAN. Each sub-interface is configured with 802.1Q encapsulation and a gateway IP address.

## Enable Physical Interface

The parent interface FastEthernet 0/0 must first be brought up with no shutdown. No IP address is assigned to the physical interface itself — all addressing is done at the sub-interface level.

enable

configure terminal interface fastEthernet 0/0 no shutdown

exit

## Sub-interface Configuration — VLAN 10

Sub-interface FastEthernet 0/0.10 is created and associated with VLAN 10 using 802.1Q encapsulation. It is assigned the default gateway address for VLAN 10.

interface fastEthernet 0/0.10 encapsulation dot1Q 10

ip address 192.168.10.150 255.255.255.0

exit

## Sub-interface Configuration — VLAN 20

Sub-interface FastEthernet 0/0.20 is similarly configured for VLAN 20, with its own unique gateway address within the 192.168.20.0/24 subnet.

interface fastEthernet 0/0.20 encapsulation dot1Q 20

ip address 192.168.20.150 255.255.255.0

exit

## Save Configuration

The running configuration is saved to NVRAM to ensure settings persist across reboots.

end

write memory

## Verification Commands

The following commands verify that the router's sub-interfaces are active and that connected routes have been installed in the routing table.

| Command | Purpose |
| --- | --- |
| show ip interface brief | Confirms sub-interfaces are up/up with correct IPs |
| show running-config ¦ section interface FastEt | heRrenveietw0s/t0he full sub-interface config |
| show ip route | Verifies directly connected routes for both VLANs |

# PC Configuration

Each end device is manually assigned a static IP address, subnet mask, and default gateway according to its VLAN membership. The gateway address corresponds to the router sub-interface IP for that VLAN, enabling cross-VLAN forwarding.

| Device | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC1 | VLAN 10 | 192.168.10.x | 255.255.255.0 | 192.168.10.150 |
| PC2 | VLAN 10 | 192.168.10.x | 255.255.255.0 | 192.168.10.150 |
| PC3 | VLAN 20 | 192.168.20.x | 255.255.255.0 | 192.168.20.150 |
| PC4 | VLAN 20 | 192.168.20.x | 255.255.255.0 | 192.168.20.150 |

# Testing and Verification

## Same-VLAN Communication (Intra-VLAN)

To validate intra-VLAN connectivity, a ping was initiated from PC1 to PC2, both of which reside in VLAN

10. The test returned 0% packet loss, confirming that access port assignments and VLAN configurations on the switch are functioning correctly. Traffic in this case does not traverse the router — it is switched entirely within VLAN 10.

## Cross-VLAN Communication (Inter-VLAN)

A ping was then executed from a device in VLAN 10 to a device in VLAN 20. Successful replies confirmed that the router is correctly processing and forwarding traffic between the two subnets. The 802.1Q-tagged frames are received on the trunk link, processed by the appropriate sub-interface, and routed to the destination subnet.

## Traceroute Verification

A traceroute from VLAN 10 to VLAN 20 was performed to trace the actual path of the packets. The output revealed a single hop through the router's sub-interface (192.168.10.150), confirming that inter-VLAN traffic is routed as expected and that the Router-on-a-Stick architecture is working correctly.

# Conclusion

This lab demonstrated a complete and functional implementation of Inter-VLAN routing using the Router-on-a-Stick technique. VLAN 10 and VLAN 20 were successfully created and named on the switch, with access ports correctly assigned to each VLAN.

A trunk link was configured between the switch (Fa0/6) and the router (Fa0/0), allowing both VLANs to share a single physical uplink while maintaining logical separation through 802.1Q tagging. On the router side, two sub-interfaces were configured — one per VLAN — each with its own encapsulation setting and gateway IP address.

All end devices were assigned appropriate IP addresses and default gateways. Connectivity tests confirmed successful communication both within the same VLAN (intra-VLAN) and across VLANs (inter-VLAN). The traceroute output provided definitive proof that inter-VLAN traffic passes through the router, validating the correct operation of the Router-on-a-Stick configuration in its entirety.

Lab 5 — Inter-VLAN Routing (Router-on-a-Stick) | Network Configuration Laboratory